ANSI/AGMA 2101-E25

Fundamental Rating Factors and Calculation Methods for Involute Spur and Helical Gear Teeth

with geometry factors per AGMA 908-B89

Implementation Findings

AGMAStandards repository — AGMA2101E25Calc project — 2026-10-16

# 1. Overview

This document records the findings from implementing ANSI/AGMA 2101-E25, Fundamental Rating Factors and Calculation Methods for Involute Spur and Helical Gear Teeth (metric edition, approved July 31, 2025, superseding ANSI/AGMA 2101-D04), as a standalone WinForms calculator (AGMA2101E25Calc) in the AGMAStandards repository, following the pattern of the AGMA 929-B22, AGMA 2003-D19, ISO 10300, ISO 23509 and AGMA 6134-C21 calculators already in the repository.

ANSI/AGMA 2101-E25 delegates the geometry factors for macropitting (Z_I) and bending (Y_J) to AGMA 908-B89, Geometry Factors for Determining the Pitting Resistance and Bending Strength of Spur, Helical and Herringbone Gear Teeth (clause 10.3). The analytical method of AGMA 908-B89 (clauses 3 to 6) was therefore implemented as well, so that the calculator rates a gear pair from its tooth and generating-tool geometry rather than requiring the geometry factors as inputs.

Scope, as agreed before implementation: the service factor method (clause 7.1) and the safety factor method (clause 7.2) for external spur and helical gear pairs, with every influence factor they use. Validation: all 44 values the Annex B illustrative example publishes for these two methods are reproduced (44/44 pass), and the AGMA 908 analytical I and J factors reproduce 24 of 24 transcribed points of the 908 geometry factor tables (section 7).

This calculator is stage 1 of a two-stage task; stage 2 ports the calculation into IGDv4.0 as a new gear set analysis (Rating > AGMA 2101-E25). Section 9 lists what was done here to make that port straightforward.

# 2. What the two documents cover

ANSI/AGMA 2101-E25 rates the macropitting resistance and the root-fillet bending strength of internal and external spur and helical involute gear teeth on parallel axes. It offers four rating methods sharing the same influence factors: service factor (7.1), safety factor (7.2), fatigue life by Miner's rule (7.3, Annex D) and the K factor / unit load method (7.4, Annex C). Compared with 2101-D04 it adds an extended discussion of safety and service factors, the reverse bending factor Y_I (clause 22) and the mesh load factor K_gamma (clause 21), removes the unity size and surface condition factors, and renames pitting to macropitting.

AGMA 908-B89 gives the analytical I and J factors for external (I also internal) spur, helical and herringbone gears generated by rack-type tools or shaper cutters: line-of-action geometry and load sharing (clause 3), the pitting geometry factor I (clause 4), the bending geometry factor J by the Errichello virtual-gear / Lewis-parabola method with Dolan-Broghamer stress correction (clause 5), addendum modification from tooth thickness (clause 6), and pre-computed tables (clause 7).

# 3. Repository changes

A new project, AGMA2101E25Calc, was added alongside the existing calculators:

- Models/Enums.cs — GearMaterial, MaterialGrade, GeneratingToolType, LoadSpecification, GeometryFactorSource, DynamicFactorMethod, LoadDistributionMethod, LeadModification, MeshAlignmentCurve, ElasticCoefficientSource, StressCycleCurve, ReliabilityRequirement.

- Models/AgmaInputs.cs — all inputs, bound to a PropertyGrid in seven categories (pair geometry, operating conditions, K_V, K_H, geometry factors and Z_E, pinion, wheel), defaulted to the Annex B example.

- Models/Examples.cs — three built-in examples: Annex B; a helical and a spur pair reproducing the geometry of two AGMA 908 tables.

- Calc/Agma908GeometryFactors.cs — AGMA 908-B89 clauses 3, 4, 5 and the clause 7.7 undercut criterion (I, J and every intermediate value).

- Calc/Agma2101Factors.cs — the ANSI/AGMA 2101-E25 influence factors as pure functions: K_V (clause 11), Z_E (12), Z_W (13), K_H method B (14), sigma_HP / sigma_FP (15, Tables 4-7, Figures 7-10), Z_N / Y_N (17), Y_Z (18), Y_B (Annex A).

- Calc/AgmaCalculator.cs — the calculation engine (clauses 6.7, 7.1, 7.2, 7.5, 8-22, Annex A) and the clause 1.2 scope checks.

- Calc/CalcTrace.cs — records every intermediate value (symbol, equation, description, value, unit) for the report, same as the other calculators.

- Calc/SelfTest.cs and Calc/SelfTest908.cs — the Annex B check, the 908 table check and the example checks (run with "AGMA2101E25Calc.exe --selftest"; exit code = number of failures). "AGMA2101E25Calc.exe --report N" prints the full report of example N to the console.

- MainForm.cs / MainForm.Designer.cs / Program.cs — the WinForms UI (PropertyGrid input, text report, example selector, Load Example, Save Report), matching the other calculators.

The hub and solution were updated:

- AGMAStandardsHub/StandardCatalog.cs — new StandardEntry for "ANSI/AGMA 2101-E25".

- AGMAStandardsHub/AGMAStandardsHub.csproj — ProjectReference to AGMA2101E25Calc.

- AGMAStandards.sln — the new project and its build configurations.

The whole solution builds with 0 warnings and 0 errors (dotnet build, Debug, net8.0-windows).

# 4. Equations implemented, by clause

| Clause | Content | Equations |
| --- | --- | --- |
| 2101 6.7, 7.5 | Gear ratio, operating pitch diameter, net face width, pitch line velocity, torque / power, tangential load | Eq 1-3, 18, 19, 25 |
| 2101 7.1 | Service factor method: allowable power at unity service factor (pinion, wheel), CSF, KSF | Eq 4-11 |
| 2101 7.2 | Safety factor method: contact and bending stress numbers, SH, SF, minimum check | Eq 12-17 |
| 2101 8, 20, 21, 22 | Overload KO, temperature Y_theta, mesh load K_gamma (inputs); reverse bending Y_I = 0.7 / 1.0 | Clause values |
| 2101 10 | Z_I and Y_J from AGMA 908-B89 or user values | see 908 rows |
| 2101 11 | Dynamic factor method B, end-point velocity, A_v from f_pt; method A / very accurate gearing as user value | Eq 21-24 |
| 2101 12 | Elastic coefficient: Table 2 pairs or Eq 26 rounded to 3 significant figures | Eq 26, Table 2 |
| 2101 13 | Hardness ratio factor: through hardened pair; surface hardened pinion with through hardened wheel | Eq 27-29 |
| 2101 14 | Load distribution factor method B (K_Hmc, K_Hpf, K_Hpm, K_Hma, K_He); method A as user value | Eq 30-35, Table 3 |
| 2101 15 | Allowable stress numbers for steel (through hardened, flame/induction A/B, carburized, four nitrided families, Grades 1-3), gray and ductile iron, bronze | Tables 4-7, Figures 7-10 |
| 2101 17 | Stress cycles; Z_N (nitrided / other) and Y_N (160/250/400 HBW, case carb, nitrided, general / critical) | Eq 49, Tables 12, 13 |
| 2101 18 | Reliability factor | Table 14 |
| 2101 Annex A | Rim thickness factor from the backup ratio | Eq A.1, Figure A.1 |
| 908 3 | Line-of-action distances C1-C6, contact ratios, minimum contact line length, load sharing ratio, operating helix and normal pressure angles | Eq 3.1-3.28 |
| 908 4 | Pitting resistance geometry factor I (conventional helical, spur and LACR), helical overlap factor | Eq 4.1-4.11 |
| 908 5 | Bending geometry factor J: virtual spur gear, load point (tip, HPSTC, LACR), tool geometry incl. shaper cutters, protuberance and pre-grind stock, generating pressure angle, Newton iteration for the critical point, fillet radius, C_h, K_f, K_psi, Y | Eq 5.1-5.78 |
| 908 7.7, 7.8 | Undercut criterion (warning); top land thickness (note / pointed tooth warning) | Eq 7.8 |

# 5. Design decisions and interpretations

## 5.1 The 908 solver and the factor library are written for the IGDv4.0 port

IGDv4.0 targets .NET Framework 4.8 and C# 7.3. Agma908GeometryFactors.cs, Agma2101Factors.cs and Enums.cs therefore use block namespaces, plain classes and no C# 8+ syntax, so they can be copied into IGD unchanged. The calculator, inputs, trace and UI stay in the modern style of the other projects in this repository.

## 5.2 J for the wheel: clause 5 applied with the roles exchanged

AGMA 908 clause 5 is written for the pinion and says to repeat it for the gear. The solver evaluates one routine for each member with the member and its mate swapped. For spur gears loaded at the HPSTC this gives, for the wheel, C1' + p_b measured from the wheel's own base-circle tangent point (equal to C6 - C2), i.e. the wheel's HPSTC is the pinion's LPSTC, as required.

## 5.3 LACR helical gears use the more conservative of two J values

Clause 5 notes that J for low axial contact ratio (m_F <= 1) helicals can exceed the conventional value, and recommends also computing J as a conventional helical with F slightly greater than p_x. The solver does both (F = 1.001 p_x for the conventional case) and uses the smaller J; both sets of intermediates appear in the report. Z_I uses the C_psi factor of Eq 4.8 and is continuous across m_F = 1 (0.1731 at m_F = 0.99, 0.1735 at 1.01 on the helical example).

## 5.4 Critical point iteration safeguarded by bisection

Eq 5.63 is solved by Newton's method from alpha_n = pi/4 with the analytical derivative of Eq 5.64 (5-6 iterations to |y| < 1e-12 on the 908 table cases). If Newton leaves (0, pi/2) or fails to converge, the solver scans downward from pi/2 for the first sign change (the correct root of Figure 5-9, not the spurious one near 0) and bisects. The fallback was needed in 0 of 48 J evaluations of the 908 table cases.

## 5.5 Z_E: Table 2 value preferred over Eq 26

Eq 26 with E = 2.05e5 N/mm^2 and nu = 0.3 gives 189.35, which rounds to 189 at three significant figures as clause 12 requires, but Table 2 lists 190 for steel on steel and Annex B uses 190. The default source is therefore Table 2 for the five pairs it lists, falling back to Eq 26 for other pairs (e.g. bronze, where E and nu must be entered). Eq 26 and a user value are selectable.

## 5.6 Y_N curves: flame/induction hardened steel and intermediate through hardness

Table 13 has curves for 400, 250 and 160 HBW, case carburized and nitrided steel only. Flame and induction hardened steel use the case carb curve (surface hardened, 50 HRC minimum). For through hardened steel at other hardness values, the 1e3-cycle value is interpolated logarithmically (ln Y_N linear in HB) between the neighbouring curves and joined by a straight log-log line to Y_N = 1.04 at 3e6 cycles, following the paragraph below Figure 18. At exactly 160, 250 or 400 HBW the Table 13 equations are used directly; below 160 and above 400 HBW the end curves are used.

## 5.7 Flank tolerance class A_v

Eq 21/22 are defined for integer 6 <= A_v <= 12. A_v from Eq 24 is computed for both members and the larger one rounded up. A_v below 6 is raised to 6 with a warning (pointing to the 1.02-1.11 'very accurate gearing' range of 11.2.1, which is a user K_V); A_v above 12 is an error. Exceeding v_t max (Eq 23) is a warning.

## 5.8 The clause 14.2 caution is reported, not enforced

The caution 'if b/d_w1 > 2.4 - 0.29 K the empirical K_H may not be conservative' uses the contact load factor K of Eq C.1. Applied literally in SI units it flags the standard's own Annex B example (K = 9.8128 N/mm^2, threshold -0.45 against b/d_w1 = 1.08), so the value is shown as a note in the report rather than as a warning. The b/d_w1 <= 2.0 and b < 1020 mm limits of method B are warnings.

## 5.9 Center distance and geometry inputs

The operating center distance defaults to the tight-mesh value computed from x1 + x2 (involute function of the operating pressure angle); an entered a_w is used as is, with a note if it differs from the tight-mesh value. Tip diameters default to m_n (z/cos beta + 2 (h_a* + x)) and can be overridden (e.g. for tip shortening). Tooth thinning for backlash enters the J calculation through the generating rack shift x_g (908 Eq 5.19), and the generating tool is described per member by its addendum, tip radius, protuberance, grinding stock and, for shaper cutters, tooth number and addendum modification.

## 5.10 User-specified geometry factors skip the geometry-based scope checks

Annex B gives Z_I and Y_J as inputs, without profile shifts or tool data, so its operating center distance (182.88 mm against 178.78 mm standard) cannot be reproduced geometrically. With user-specified geometry factors the AGMA 908 calculation is skipped entirely, and the clause 1.2 checks that need it (contact ratio limits, interference, undercut, pointed teeth) are reported as not performed.

## 5.11 Materials

Each member selects a material/heat treatment, a metallurgical grade and the relevant hardness: Brinell (through hardened surface, or nitrided core for Figures 9 and 10), HRC (flame/induction rows, carburized minimums, the 48 HRC test of 13.2) or HR15N (nitrided rows). Hardness below a Table 4 row minimum is a warning and the lowest row is used; hardness outside the plotted range of Figures 7-10 is a warning (extrapolated). A grade not defined for the material (e.g. Grade 3 through hardened) is an error. For iron and bronze the lower value of each Table 6/7 range is used unless 'upper value' is selected (note 3), and Z_N = Y_N = 1.0 (17.2). The bainite option of Table 5 note 6 (carburized Grade 2 at 485 N/mm^2) is available. Allowable stress numbers can also be entered directly.

## 5.12 Service factor method uses K_O = 1 and Y_Z = 1

Eq 6-10 contain neither K_O nor Y_Z, consistent with 7.1 (unity service factor corresponds to K_O = 1, Y_Z = 1). K_O and Y_Z are used only in the safety factor method (Eq 12-17). With K_O = Y_Z = 1, SF equals KSF and SH^2 equals CSF, which the report shows.

## 5.13 Load for the required safety factors (added 2026-10-16)

At user request the calculator also gives the load a gear set can transmit for required safety factors: the input 'Rating goal' is CheckTransmittedLoad (the safety and service factors of the load given, as before) or RatedLoadForRequiredSafetyFactors, and 'Load as' chooses whether that load is given as a power or as a pinion torque (the other is reported too). At a given pinion speed and life no factor of the rating depends on the load (K_V on the pitch line velocity, K_H method B on the geometry and mounting, Z_N and Y_N on the number of cycles, the geometry factors on the geometry), while sigma_H grows with the square root of the load and sigma_F with the load. Setting S_H = S_Hmin and S_F = S_Fmin therefore gives, for each member, P_H = P_azu / (K_O (S_Hmin Y_Z)^2) and P_F = P_ayu / (K_O S_Fmin Y_Z) in closed form from the allowable powers of 7.1, and the load for the required safety factors is the smallest of the four, T = 30000 P / (pi n1). The report shows the four powers, the governing member and mode, and the load; in the rated goal the whole rating is then made at that load (its powers are taken from a first pass at 1 kW, which they do not depend on), so the governing safety factor equals the one required. The section is in the report in both goals. The load holds only for the speed, life and K_O given, and the yield check under peak loads (clause 19) is still not made.

# 6. Not implemented (by design)

- Fatigue life by Miner's rule (7.3, Annex D), the K factor / unit load method (7.4, Annex C) and the yield strength check (clause 19, Annex E): outside the agreed scope. The Annex C contact load factor K is computed only for the clause 14.2 caution note.

- Internal gears: the agreed scope is external gears only. 2101-E25 clause 10.4 requires the bending capacity of internal gears to be calculated to ISO 6336-3, not with Y_J.

- Case depth guidance (clause 16) and the metallurgical requirement tables (Tables 8-11): qualitative / specification content, not rating equations.

- Method A determinations of K_V and K_H (measurement, AGMA 927 or ISO 6336-1 analysis): entered as user values.

- Double helical gears as a unit: 7.7 treats each helix as a single helical gear carrying its share of the torque; rate one helix with its own face width and torque.

- AGMA 908 Appendix F (transverse contact ratio above 2): 2101-E25 does not apply above 2.0, which is a warning.

# 7. Validation

## 7.1 ANSI/AGMA 2101-E25 Annex B

Annex B (B.1) is reproduced from its own input data with Z_I = 0.231, Y_J = 0.505 / 0.521 and Z_E = 190 as given. All 44 published values of the service and safety factor methods pass (44/44). The tolerance is 1e-6 relative with an absolute floor of half a unit in the sixth printed decimal, widened only where the standard's own arithmetic carries rounded intermediates (noted).

| Symbol | Annex B | Calculated | Diff % | Result | Note |
| --- | --- | --- | --- | --- | --- |
| beta | 0.191986 | 0.191986 | 0.0001 | PASS |  |
| u | 4.2 | 4.2 | 0.0000 | PASS |  |
| n2 | 345.238095 | 345.238095 | 0.0000 | PASS |  |
| d1 | 68.763374 | 68.763377 | 0.0000 | PASS |  |
| d2 | 288.806171 | 288.806183 | 0.0000 | PASS |  |
| dw1 | 70.338462 | 70.338462 | -0.0000 | PASS |  |
| vt | 5.340224 | 5.340224 | 0.0000 | PASS |  |
| Ft | 42480.314682 | 42480.314961 | 0.0000 | PASS |  |
| P | 226.854406 | 226.854406 | -0.0000 | PASS |  |
| B | 0.25 | 0.25 | 0.0000 | PASS |  |
| C | 92 | 92 | 0.0000 | PASS |  |
| KV | 1.078395 | 1.078395 | -0.0000 | PASS |  |
| KHpf | 0.108324 | 0.108324 | -0.0002 | PASS |  |
| KHpm | 1.1 | 1.1 | 0.0000 | PASS |  |
| KHma | 0.105069 | 0.105069 | -0.0003 | PASS |  |
| KHbeta | 1.17938 | 1.17938 | -0.0000 | PASS | printed to five decimals |
| KH | 1.17938 | 1.17938 | -0.0000 | PASS | printed to five decimals |
| nL1 | 8.7000E+8 | 8.7000E+8 | 0.0000 | PASS |  |
| nL2 | 2.0710E+8 | 2.0714E+8 | 0.0207 | PASS | printed to four significant figures |
| ZE | 190 | 190 | 0.0000 | PASS | Table 2 value |
| sigma_HP1 | 1550 | 1550 | 0.0000 | PASS |  |
| sigma_HP2 | 1550 | 1550 | 0.0000 | PASS |  |
| sigma_FP1 | 450 | 450 | 0.0000 | PASS |  |
| sigma_FP2 | 450 | 450 | 0.0000 | PASS |  |
| ZN1 | 0.902401 | 0.902401 | -0.0000 | PASS |  |
| ZN2 | 0.932688 | 0.932683 | -0.0005 | PASS | B.1 evaluates it with n_L2 rounded to 2.071e8 |
| YN1 | 0.93988 | 0.93988 | -0.0001 | PASS | printed to five decimals |
| YN2 | 0.964201 | 0.964198 | -0.0004 | PASS | B.1 evaluates it with n_L2 rounded to 2.071e8 |
| Pazu1 | 281.736437 | 281.736399 | -0.0000 | PASS | B.1 substitutes the rounded KV, KH |
| Pazu2 | 300.965462 | 300.962528 | -0.0010 | PASS | B.1 substitutes the rounded KV, KH, ZN2 |
| Pazu | 281.736437 | 281.736399 | -0.0000 | PASS |  |
| CSF | 1.241926 | 1.241926 | 0.0000 | PASS |  |
| Payu1 | 313.274509 | 313.274429 | -0.0000 | PASS | B.1 substitutes the rounded KV, KH |
| Payu2 | 331.563391 | 331.562289 | -0.0003 | PASS | B.1 substitutes the rounded KV, KH, YN2 |
| Payu | 313.274509 | 313.274429 | -0.0000 | PASS |  |
| KSF | 1.38095 | 1.380949 | -0.0001 | PASS |  |
| sigma_H | 1255.114556 | 1255.114427 | -0.0000 | PASS |  |
| SH1 | 1.114417 | 1.114417 | 0.0000 | PASS | Eq 13 line of B.1 repeats it as 1.114517 (typo) |
| SH2 | 1.15182 | 1.151815 | -0.0005 | PASS |  |
| sigma_F1 | 306.271849 | 306.271772 | -0.0000 | PASS |  |
| sigma_F2 | 296.866188 | 296.866113 | -0.0000 | PASS |  |
| SF1 | 1.38095 | 1.380949 | -0.0001 | PASS |  |
| SF2 | 1.461569 | 1.461564 | -0.0003 | PASS |  |
| P_SF | 281.736437 | 281.736399 | -0.0000 | PASS | SHmin = SFmin = 1, KO = YZ = 1: min(P_azu, P_ayu) of B.1 |

## 7.2 AGMA 908-B89 geometry factor tables

The 908 tables were computed with the method of clause 5 for a hob (n_c = 10 000, s_n0 = pi/2, x0 = 0, no protuberance), standard center distance, tip diameters per Eq 7.1/7.2, thinning 0.024 per member (Eq 7.5) and m_F = 2.0 for helicals (clause 7.10). 24 points across six tables (spur HPSTC with x = 0 and +/-0.25; 30 deg helix with x = +/-0.25 and +/-0.50; 14.5 deg / 20 deg helix with x = 0 and +/-0.25) were transcribed. I is printed to three decimals and checked to +/-0.0006; J is printed to two decimals and checked to +/-0.006. 24/24 pass. Table cells show printed / calculated.

| Table | PA / helix / x1 | z1/z2 | I | J pinion | J wheel | Result |
| --- | --- | --- | --- | --- | --- | --- |
| p.41 top | 20 / 0 / 0 | 21/21 | 0.078 / 0.0778 | 0.33 / 0.3259 | 0.33 / 0.3259 | PASS |
| p.41 top | 20 / 0 / 0 | 21/35 | 0.091 / 0.0914 | 0.34 / 0.3360 | 0.37 / 0.3714 | PASS |
| p.41 top | 20 / 0 / 0 | 26/55 | 0.101 / 0.1011 | 0.37 / 0.3676 | 0.41 / 0.4058 | PASS |
| p.41 top | 20 / 0 / 0 | 35/135 | 0.120 / 0.1201 | 0.41 / 0.4100 | 0.45 / 0.4488 | PASS |
| p.41 top | 20 / 0 / 0 | 55/55 | 0.080 / 0.0802 | 0.43 / 0.4276 | 0.43 / 0.4276 | PASS |
| p.41 top | 20 / 0 / 0 | 135/135 | 0.080 / 0.0803 | 0.49 / 0.4856 | 0.49 / 0.4856 | PASS |
| p.41 top | 20 / 0 / 0 | 21/135 | 0.118 / 0.1176 | 0.35 / 0.3530 | 0.43 / 0.4296 | PASS |
| p.41 bottom | 20 / 0 / +0.25 | 17/17 | 0.080 / 0.0797 | 0.36 / 0.3616 | 0.24 / 0.2353 | PASS |
| p.41 bottom | 20 / 0 / +0.25 | 21/35 | 0.098 / 0.0985 | 0.40 / 0.3954 | 0.32 / 0.3214 | PASS |
| p.41 bottom | 20 / 0 / +0.25 | 35/135 | 0.129 / 0.1287 | 0.45 / 0.4537 | 0.42 / 0.4162 | PASS |
| p.41 bottom | 20 / 0 / +0.25 | 55/55 | 0.080 / 0.0803 | 0.47 / 0.4653 | 0.39 / 0.3863 | PASS |
| p.49 top | 20 / 30 / +0.25 | 12/12 | 0.115 / 0.1151 | 0.42 / 0.4246 | 0.30 / 0.3029 | PASS |
| p.49 top | 20 / 30 / +0.25 | 21/35 | 0.174 / 0.1741 | 0.49 / 0.4885 | 0.44 / 0.4384 | PASS |
| p.49 top | 20 / 30 / +0.25 | 17/55 | 0.224 / 0.2236 | 0.48 / 0.4798 | 0.46 / 0.4578 | PASS |
| p.49 top | 20 / 30 / +0.25 | 135/135 | 0.151 / 0.1506 | 0.57 / 0.5681 | 0.55 / 0.5480 | PASS |
| p.49 top | 20 / 30 / +0.25 | 26/26 | 0.133 / 0.1331 | 0.50 / 0.4963 | 0.42 / 0.4213 | PASS |
| p.49 bottom | 20 / 30 / +0.50 | 12/12 | 0.098 / 0.0984 | 0.47 / 0.4727 | 0.24 / 0.2362 | PASS |
| p.49 bottom | 20 / 30 / +0.50 | 21/135 | 0.274 / 0.2742 | 0.52 / 0.5211 | 0.47 / 0.4697 | PASS |
| p.49 bottom | 20 / 30 / +0.50 | 55/55 | 0.141 / 0.1411 | 0.55 / 0.5517 | 0.47 / 0.4672 | PASS |
| p.35 top | 14.5 / 20 / 0 | 35/35 | 0.125 / 0.1251 | 0.43 / 0.4265 | 0.43 / 0.4265 | PASS |
| p.35 top | 14.5 / 20 / 0 | 35/135 | 0.215 / 0.2155 | 0.46 / 0.4626 | 0.49 / 0.4886 | PASS |
| p.35 top | 14.5 / 20 / 0 | 135/135 | 0.146 / 0.1463 | 0.53 / 0.5267 | 0.53 / 0.5267 | PASS |
| p.35 bottom | 14.5 / 20 / +0.25 | 21/21 | 0.104 / 0.1039 | 0.40 / 0.4055 | 0.32 / 0.3176 | PASS |
| p.35 bottom | 14.5 / 20 / +0.25 | 26/55 | 0.183 / 0.1830 | 0.44 / 0.4452 | 0.41 / 0.4081 | PASS |

22 of the 24 points round exactly to the printed values. The two exceptions are the pinion J of the 14.5 deg tables for 21/21 and 26/55 at x = +/-0.25: 0.4055 and 0.4452 against printed 0.40 and 0.44, 0.0005 past the rounding boundary. Rounding H, L and M of Eq 5.74-5.76 to two decimals (which clause 5.11 permits) does not explain them and moves the other tables away from their printed values, so the unrounded coefficients are kept; the J tolerance of 0.006 covers these two points.

## 7.3 Built-in examples

The helical (21/35, 30 deg, x = +/-0.25, m_n = 5 mm, b = 2 p_x) and spur (21/35, HPSTC, x = 0, m_n = 4 mm) examples carry the computed Z_I and Y_J through the full rating and reproduce the 908 table values (0.174, 0.49/0.44 and 0.091, 0.34/0.37). Their material and operating data are illustrative, not from a published source.

## 7.4 Load for the required safety factors

Annex B gives four reference points for the inverse rating: with S_Hmin = S_Fmin = 1 (K_O = Y_Z = 1) the power is min(P_azu, P_ayu) = 281.736437 kW; with S_Hmin = S_H1 of B.1 (1.114417) the pinion's macropitting gives back B.1's own power, 226.854406 kW, and as a pinion torque its own input torque, 1494 Nm; with S_Fmin = S_F1 of B.1 (1.38095) its bending gives the same power. The residual differences come from the six printed digits of S_H1 and S_F1. On every built-in example rated for S_H = 1.3, S_F = 1.6, the rating at the load found has its governing safety factor equal to the one required to 1e-9.

| Case | Reference | Calculated | Diff % | Result |
| --- | --- | --- | --- | --- |
| Annex B, S_Hmin = S_Fmin = 1: power [kW] | 281.736437 | 281.736399 | -0.0000 | PASS |
| Annex B, S_Hmin = 1.114417: power [kW] | 226.854406 | 226.854554 | 0.0001 | PASS |
| Annex B, S_Hmin = 1.114417: pinion torque [Nm] | 1494 | 1494.00098 | 0.0001 | PASS |
| Annex B, S_Fmin = 1.38095: power [kW] | 226.854406 | 226.854288 | -0.0001 | PASS |

## 7.5 Paths without published reference values

The following were exercised for plausibility only, as no reference values exist in either document: LACR helicals (continuity across m_F = 1), shaper cutters (n_c = 30, x0 = 0.2), pre-grind protuberance, spur tip loading, gray/ductile iron and bronze pairs, Z_W by Eq 29, Y_N interpolation at intermediate Brinell hardness, A_v from f_pt, undercut and interference detection (12-tooth pinion) and top land thickness. Independent cross-checks of these paths against other software (e.g. KISSsoft, as done for ISO 10300) are recommended.

# 8. Observations on the standards

- 2101-E25 Annex B, Eq 13 line: min(SH1, SH2) is printed as 1.114517; SH1 two lines above is 1.114417, which the calculation reproduces.

- 2101-E25 Annex B evaluates Z_N2 and Y_N2 with n_L2 rounded to 2.071e8 (exact 2.0714e8), a 5e-6 relative effect.

- 2101-E25 Table 2 lists Z_E = 190 for steel on steel, while Eq 26 rounded to three significant figures as clause 12 requires gives 189.

- 2101-E25 Table 1 (symbols) is internally misaligned in the published PDF (descriptions, units and references shifted between rows); the equations and their where-lists were used instead.

- 2101-E25 Table 13 gives no Y_N curve for flame or induction hardened steel (see 5.6).

- 2101-E25 Table 12 lists the nitrided critical service range as 1e7 <= n_L < 1e10 and the general range as 1e7 < n_L < 1e10; both are continuous with the 1e4-1e7 branch at 1e7 (Z_N = 1.000).

- AGMA 908-B89 Eq 5.49 writes the exponent of the arc-involute starting value as 0.33; the solver uses 1/3, which only affects the starting point of the iteration.

# 9. Notes for the IGDv4.0 port (stage 2)

- Port Agma908GeometryFactors.cs, Agma2101Factors.cs and Enums.cs unchanged into the IGD analysis project; the calculator logic of AgmaCalculator.Calculate maps onto an analysis ComputeAsync.

- IGD gear geometry maps directly onto the 908 inputs: Module, PressAng, HelixAngle, TeethNum, FaceWidth, ProfShiftCoef, GeneratingShiftCoef (x_g), OperatingCenterDistance, AddenCoef, and the rack cutter's DedenCoef (tool addendum) and EdgeRadiusCoef (tool tip radius).

- IGD has no gear material database: the material, grade, hardness, E and nu inputs become per-analysis variables, as the LTCA does for E and nu.

- Only external spur and external helical gear sets are ported in IGDv4.0, matching the external-only scope; the new analysis is offered only for those types, under a nested Add new analysis > Rating > AGMA 2101-E25 menu.

- The Annex B and 908-table checks of SelfTest become NUnit tests in IGDAnalysis.Tests and reference checks in the IGD technical report, per the IGD repository conventions.