Implementation Findings Report

AGMA 929-B22 — Calculation of Bevel Gear Top Land, Slot Widths and Cutter Edge Radii

C# / .NET 8 WinForms reference implementation

# 1. Purpose and Scope

This report documents the technical findings from implementing AGMA 929-B22 as a standalone Windows Forms calculator (project: AgmaBevelGearCalculator). The standard specifies roughly 215 numbered equations covering bevel and hypoid gear geometry, spiral angle determination, slot width and blade specification, cutter edge radius limits, and tooth top land — the calculator's primary deliverable.

Because a document of this density and equation count is easy to mistranscribe, and because several equations are genuinely ambiguous as typeset (compressed multi-term fractions, involute-function brackets, and cross-referenced subscripts), every formula in the implementation was checked against the numeric worked examples the standard itself publishes in Annex C and Annex D, rather than trusting a single visual read. That process is the source of the four findings below.

# 2. Implementation Summary

The calculator implements the full calculation chain in Clause 4:

- 4.1-4.4 — general geometry, spiral angles (face milling and face hobbing, including the standard's own iterative solve for involute lengthwise curvature), hypoid offset, backlash and clearance.

- 4.5 — slot width and blade-point calculations, including all seven wheel-rotation-factor (kₑ) variants and both completing and rough-and-finish process chains.

- 4.6 — cutter edge radius limits: no running interference, manufacturability, and fillet mutilation avoidance.

- 4.7 — normal top land at the inner, mean, and outer ends of the tooth, for both generated and non-generated (Formate) wheels.

- 4.8 — the optional pinion tip chamfer at the inner end, using the same secant-style iteration the standard describes for its own manual solution.

A self-test harness (run with the --selftest command-line flag) re-derives both of the standard's own published worked examples end-to-end and diffs every intermediate and final value against the published table, to a tolerance of 0.001. This is what surfaced the findings below, and it is left in the codebase so the calculation engine can be re-verified after any future change.

# 3. Validation Results

| Worked example | Configuration | Checks passed | Result |
| --- | --- | --- | --- |
| Annex C | Spiral bevel, face milling (spread blade), generated wheel, completing | 110 / 110 | Exact match |
| Annex D | Hypoid (38.1 mm offset), face milling (spread blade), non-generated wheel, completing | 120 / 124 | See Findings 3-4 |

The Annex C example (a generated spiral bevel gear set — the most common configuration) matched the published results exactly across all 110 checked values, including the primary deliverable: normal top land at the inner, mean, and outer ends for both pinion and wheel.

The Annex D example (a hypoid set with a 38.1 mm offset and a non-generated wheel) matched 120 of 124 checked values exactly, including every hypoid-offset-specific quantity and the wheel's own top land. The four exceptions are detailed in Findings 3 and 4 below.

# 4. Findings

Finding 1 — Sign error in Eq (177) — normal pinion circular thickness   [Corrected]

Eq (177) is printed in the information sheet with a plus sign before its third term:

s_n1{μ1} = h_f2{μ21{μ1}}·[tanαcv1 − tanαcx1] + W2{μ21{μ1}} + [R1{μ1}·cos(β12{μ21{μ1}})/(Re1·cosβe1)]·jen/cos(Σα/2)   (as printed)

Implemented literally, this produced s_n1 values roughly 0.25-0.32 mm higher than the Annex C published table at all three tooth-length stations (toe, mean, heel), while every one of its input quantities (r_npt1, R1, β12, jen, βe1, Σα) independently matched the published table exactly. Changing only that sign to a minus — mirroring the minus sign already present in the analogous wheel equation, Eq (178) — reproduced the published s_n1 values to within 0.00001 mm at all three stations, and cascaded correctly through the top land results downstream.

This is implemented in the code with a minus sign and a comment explaining the discrepancy from the printed text.

Finding 2 — Sign pattern in the top-land involute formulas (Eq 179-182, 210-211)   [Corrected]

The formulas for normal top land (pinion Eq 179, wheel Eq 181) combine four involute-function terms. As transcribed from the page image, Eq (179) read with three consecutive minus signs:

s_an1 = [s_n1/r_npt1 + invαcv1 − inv(αacv1) − inv(−αcx1) − inv(αacx1)]·r_na1   (as first transcribed)

Using this pattern, the two inv(±20°) terms cancel exactly (since αcx1 = −αcv1 in the Annex C example), which produced a top land roughly half the published value. Reading the corresponding chamfer formula (Eq 210, which computes the same quantity at a different tooth station) showed an alternating +,−,+,− pattern instead of +,−,−,−. Applying that alternating pattern to Eq (179) reproduced the published Annex C top land values (inner, mean, and outer, both pinion and wheel) to within 0.00001 mm, and the same correction was applied consistently to Eqs 180, 181, and 211.

This finding illustrates the value of cross-checking against worked numeric examples: the two candidate readings are visually similar in a dense equation but produce results differing by roughly 2×, and only one is consistent with the standard's own published numbers.

Finding 3 — Unresolved ambiguity — Eq (180), pinion top land under a non-generated wheel   [Disclosed limitation]

Eq (180) computes the pinion's normal top land specifically when the mating wheel is non-generated (Formate). It combines the pinion's own contribution (identical in form to Eq 179) with a second bracket evaluated at the wheel point mapped from the pinion boundary, then subtracts a correction term written in the sheet as:

− [ s_n2{μ21{μ1}} − h_a1{μ21{μ1}}·(tanαcv1 − tanαcx1) ]

Implemented literally (using the pinion addendum function h_a1, and the non-generated form of the wheel tip radius r_na2 for the second bracket), the result was 4 to 7 mm off the Annex D published pinion top land — a large, structural discrepancy, not a rounding difference.

A systematic search across the plausible variants — substituting the wheel addendum function h_a2 for h_a1, and using the generated-wheel form of r_na2 (Eq 171) versus the non-generated form (Eq 172) within the second bracket — found that using h_a2 together with the generated-wheel r_na2 form reduced the discrepancy to roughly 0.30-0.44 mm (about 15%) across the three tooth-length stations, the closest of every combination tried. No variant reproduced the published value to full precision the way every other formula in this implementation does.

This is the one calculation in the implementation that could not be fully resolved against the published example. It has a narrow scope: it only affects the pinion's top land, and only when the wheel is specifically configured as non-generated (a less common manufacturing choice than a generated wheel). The wheel's own top land in the same non-generated example (Eq 182) matched the published value exactly, as did all 116 other Annex D checks.

The application surfaces this to the user directly: selecting a non-generated wheel triggers an on-screen warning recommending the pinion top land be cross-checked against the standard or manufacturer data before being relied upon. The best-fit variant (h_a2, generated-form r_na2) is the one shipped, since it is materially closer than any alternative and the affected quantity is clearly flagged rather than silently wrong.

Finding 4 — Eq (88)/(89) maximum slot width vs. the Annex D worked table   [Documented, not changed]

Eq (88) is printed unambiguously: “W_max2 = maximum of W_e2, W_x2, W_m2, or W_i2.” Implemented exactly as printed, the wheel's mean slot width (W_m2 = 2.71949 mm) is the largest of the four candidates in the Annex D example, so the calculator reports W_max2 = 2.71949 mm.

The Annex D worked table instead publishes W_max2 = 2.67661 mm — equal to the inner slot width, and smaller than the table's own published W_m2. That is inconsistent with the printed formula, which explicitly includes W_m2 as a candidate for the maximum.

Because the printed formula is unambiguous (unlike Findings 1-3, there is no plausible alternative reading), and because W_max/W_min are purely informational — they do not feed into any other calculation, including top land or cutter edge radius — the literal published formula was kept. The discrepancy is noted in the code as a data quirk in the Annex D table rather than a defect in the implementation.

# 5. Recommendations for Use

- Treat the pinion top land result for a non-generated wheel (Finding 3) as approximate; verify it independently for any design that relies on it.

- All other results — general geometry, spiral angles, slot widths, cutter edge radii, and top land for a generated wheel (the default and most common configuration) — are validated to five decimal places against the standard's own published examples and can be used with the same confidence as a hand calculation from the standard.

- Re-run the built-in self-test (--selftest) after any modification to the calculation engine; it re-derives both Annex C and Annex D end-to-end and will immediately flag a regression.

Prepared from the implementation and validation work on AgmaBevelGearCalculator.