ANSI/AGMA 6134-C21

Practice for Enclosed Cylindrical Wormgear Speed Reducers and Gearmotors

Implementation Findings

AGMAStandards repository — AGMA6134C21Calc project — 2026-09-19

# 1. Overview

This document records the findings from implementing ANSI/AGMA 6134-C21, Practice for Enclosed Cylindrical Wormgear Speed Reducers and Gearmotors (approved April 8, 2021), as a standalone WinForms calculator (AGMA6134C21Calc) in the AGMAStandards repository, following the same pattern established by the AGMA 929-B22, AGMA 2003-D19, ISO 10300 and ISO 23509 calculators already in the repository.

The implementation covers clause 4 (preliminary calculations), clause 5 (rating factors), clause 6 (wear and bending strength ratings and efficiency), clause 7.1.4 (overhung shaft loads), clause 10.1 (equivalent input power / service factor), clause 11.1 (recommended lubricant viscosity, Table 9) and clause 12 (self-locking). It was validated end-to-end against the standard's own Annex C worked example, which every equation in clauses 4 through 6 passes to within numerical rounding of the published values (see section 4).

# 2. What ANSI/AGMA 6134-C21 covers

The standard gives a wear and bending-strength rating method, at a nominal 1.0 service factor and 25 000 hour design life, for cylindrical wormgear meshes where the worm is surface-hardened steel (carburized 58-62 HRC, or flame/induction hardened 50-62 HRC) and the wormgear is sand, static-chill/forged, or centrifugally cast tin bronze (UNS C90700, C91700 or C67300), meshing at standard center distance, with a gear ratio z2/z1 >= 3.0, worm speed <= 3600 rpm and sliding velocity <= 30 m/s. It is the metric edition of ANSI/AGMA 6034-C21 and replaces the tabulated empirical factors of the withdrawn AGMA 6034-B92 with closed-form equations fitted to the same underlying curves, plus a new lubricant-type factor.

Clauses outside the rating equations themselves (thermal rating capacity determination, housing and bearing design per AGMA 6101, lubricant compatibility and flushing procedure, cold-start provisions) are qualitative manufacturer guidance rather than closed-form calculations, and are summarized in section 6 ("Not implemented") rather than coded.

# 3. Repository changes

A new project, AGMA6134C21Calc, was added alongside the existing calculators:

- Models/Enums.cs — GearCastingMethod, WormHeatTreat, OilType, OverhungDriveType, ServiceDuration, ShockLoadClass, AmbientTemperatureBand.

- Models/AgmaInputs.cs — all inputs, bound to a PropertyGrid, defaulted to the Annex C worked example.

- Calc/MathUtil.cs — degree-based trig helpers and the radian-argument arcsine required by Eq 12.

- Calc/CalcTrace.cs — records every intermediate value (symbol, equation #, description, value, unit) for the results report, same pattern as the other calculators.

- Calc/AgmaCalculator.cs — the calculation engine (clauses 4, 5, 6, 7.1.4, 10.1, 11.1, 12).

- Calc/SelfTest.cs — reproduces the Annex C example and checks all 27 published intermediate/final values (run with "AGMA6134C21Calc.exe --selftest").

- MainForm.cs / MainForm.Designer.cs / Program.cs — the WinForms UI (PropertyGrid input, text report output, "Load Example" and "Save Report" buttons), matching the layout of the other calculators in the repository.

The hub application was updated to list the new calculator:

- AGMAStandardsHub/StandardCatalog.cs — new StandardEntry for "ANSI/AGMA 6134-C21".

- AGMAStandardsHub/AGMAStandardsHub.csproj — added a ProjectReference to AGMA6134C21Calc.

- AGMAStandards.sln — added the new project and its build configurations.

The whole solution (AGMAStandards.sln) builds with 0 warnings and 0 errors after these changes (dotnet build, Debug configuration, net8.0-windows).

# 4. Equations implemented, by clause

| Clause | Content | Equations |
| --- | --- | --- |
| 4.2 - 4.10 | Helix angle, pitch diameters, axial pitch, center distance, gear ratio, sliding velocity, gear speed, effective face width, gear tooth root length | Eq 1, 4, 6-12 |
| 1.2 (scope) | Scope checks: worm pitch diameter range, worm maximum radial deflection, gear ratio >= 3.0, worm speed <= 3600 rpm, sliding velocity <= 30 m/s, worm hardness range (Table 1) | Eq 2, 3 |
| 5.1 | Materials factor Ks (via Kd, Table 3, by casting method and pitch diameter) | Eq 13, Table 3 |
| 5.2 | Ratio correction factor Km | Table 4 |
| 5.3 | Sliding velocity factor Kv | Table 5 |
| 5.4 | Gear speed factor Kn | Table 6 |
| 5.5 | Oil type factors Z_oil-wear, Z_oil-mu | Table 7 |
| 5.6 | Dynamic coefficient of friction, mu | Eq 14 |
| 6.1 | Wear rating chain: allowable tangential load, friction force, output power, power loss, input power rating, allowable torque | Eq 15-20 |
| 6.2 | Forward (worm-driving) and back-drive (gear-driving) efficiency | Eq 21, 22 |
| 6.3 | Bending rating chain: allowable bronze bending stress (constant), allowable tangential load, allowable torque, input power rating | Eq 23-26 |
| 7.1.4 | Overhung shaft load (optional; requires an applied input power and mounted-element pitch diameter) | Eq 27, Table 8 |
| 10.1 | Equivalent input power = specified power x service factor, compared to the governing (minimum of wear/bending) rated input power | Annex A Table A.1 |
| 11.1 | Recommended lubricant viscosity (ISO VG) by mesh center distance, worm speed and ambient band | Table 9 |
| 12.2 | Self-locking indicator from the sign of the back-drive efficiency (Eq 22) | Eq 22 |

# 5. Design decisions and interpretations

A few points in the standard required an explicit modeling decision because they are either under-specified for a general-purpose calculator, or reference material outside this standard's own scope:

## 5.1 Overhung shaft loads (Eq 27) need a torque, which needs an assumed input power

Equation 27 rates an overhung load from a shaft torque T_k, but the standard's own numeric example (Annex C) never exercises clause 7 and so never demonstrates where T_k comes from for a specific application. T1 (worm shaft) was derived from a user-supplied "applied input power" input via T = 30000*P/(pi*n), the same power/torque/speed relationship the standard itself uses in Eq 26. T2 (wormgear shaft) was derived as the corresponding output power (input power times the forward efficiency eta_z1-2 from Eq 21) converted the same way. This whole section is gated behind a "Check overhung shaft loads" checkbox (default off) and is clearly labeled optional in the UI, since it is a downstream convenience calculation built on top of the standard's equations rather than a literal transcription of one.

## 5.2 Service factor (clause 10.1) uses Table A.1, not the much larger Table A.3

Annex A gives two ways to select a service factor: Table A.1 (a small 4x4 duration/shock-load matrix, for when magnitude and duration of shock loads are estimated directly) and Table A.3 (a several-hundred-row list of named applications from "Agitators" to "Winders", each with a duration-based service factor, spanning pages 23-29 of the standard). Table A.1 was implemented directly since it is small, general, and equation-like. Table A.3 was intentionally not transcribed: it is a reference lookup table of named industrial applications rather than a calculation, and reproducing several hundred rows verbatim inside a calculator's source code would be a transcription-error risk without adding calculation logic. A "manual service factor" override is provided instead so a value read from Table A.3 (or agreed with the manufacturer) can still be used directly in the clause 10.1 comparison.

## 5.3 Worm hardness and heat treatment are scope checks only, not calculation inputs

Table 1 (worm surface hardness and typical material) and the bronze alloy/casting-method list in clause 1.2 i) define the applicability boundary of the standard, not a term in any rating equation. The calculator uses the worm heat-treatment selection only to display a pass/fail scope check against Table 1's hardness range (Eq-adjacent, clause 1.2 d)); it has no numerical effect on Ks, Km, Kv, Kn, mu, or any downstream rating value, matching the standard's own equations, none of which take a hardness or alloy term.

## 5.4 Table 9 (lubricant viscosity) transcribed exactly, including its one asymmetry

Table 9's recommended ISO VG for the colder ambient band (-10 C to 10 C) is 460 for every row. For the warmer band (10 C to 52 C) it is 680 at lower worm speeds and 460 at higher speeds, with the break speed decreasing as mesh center distance increases (700, 450, 300, 250, 200 rpm) - except that the smallest center-distance band (a <= 152 mm) recommends 680 at both its speed sub-bands. This exact behavior (including the top-band exception) was reproduced in LubricantViscosityTable9(); it is presented as an informative note in the results report, not a rating value, matching the standard's own framing of clause 11 as lubrication guidance.

## 5.5 Worm radial deflection (Eq 3) is a user-supplied check value, not calculated

Clause 1.2 h) requires the worm's maximum radial deflection under load to satisfy Eq 3, but the standard explicitly defers the deflection calculation itself to ANSI/AGMA 6022 (Design Manual for Cylindrical Worm Gearing), a separate standard not implemented here. The calculator therefore takes the deflection as a direct input (defaulted to the Annex C example's own value, 0.076 mm) and only evaluates the Eq 3 pass/fail comparison, consistent with the Annex C example itself, which does the same (C.3.6).

## 5.6 Self-locking (clause 12) is reported as a directional indicator, not a certification

The standard is explicit that a positive (non-zero) margin on Eq 22's back-drive efficiency is necessary but not sufficient for practical self-locking: vibration can unlock an otherwise statically self-locking mesh, and the standard recommends an anti-rollback device wherever self-locking must be assured (clause 12.3). The calculator's self-locking note reproduces this caveat verbatim alongside the eta_z2-1 <= 0 check, rather than presenting a bare pass/fail.

# 6. Not implemented (by design)

The following clauses are descriptive/qualitative manufacturer or user guidance rather than closed-form calculations, and were left out of the calculator on that basis:

- Clause 8 (Thermal rating) - thermal capacity is determined by test per 8.3's four qualifications (oil level, run-in, steady-state operation, measurement point); the standard gives no formula for it.

- Clause 9.1-9.2 (No-load losses, multiple reductions) - no-load loss is experimentally determined, and multi-reduction rating is a described procedure (rate each pair, take the governing one) rather than a new equation; a user modeling a specific multi-stage reducer would run this calculator once per stage.

- Clause 9.3-9.4 (Spur/helical/bevel stages, momentary overloads) - these reference ANSI/AGMA 6113 for non-wormgear stages and state qualitative overload limits (300% for up to 2 seconds, 25 000 cycles over life) with no new formula.

- Clause 11 (Lubrication) beyond Table 9 - compatibility, flushing procedure, additive restrictions, cold-start and sump-heater guidance are procedural/qualitative.

- Annex A Table A.3 (application-specific service factors) - see section 5.2.

- Annex B (User recommendations) - informative purchasing/installation guidance (protection, standard ratios, bore sizing, sound), not a calculation.

None of these omissions affect the correctness of the implemented rating chain: Annex C's own worked example does not exercise any of them either.

# 7. Validation against Annex C

Annex C's worked example (worm: 8620 steel, 58 HRC, single-start, 6 deg lead angle; wormgear: 30-tooth C90700 bronze, centrifugally cast, 6 deg helix angle; mn = 4 mm, alpha_n = 20 deg, n1 = 1750 rpm; PAO oil) was encoded exactly as published (Table C.1) and run through AgmaCalculator.Calculate(). All 27 published intermediate and final values were checked; every one passed. The largest deviation (0.73%, on the sliding velocity v1) is fully explained by the standard itself: Annex C prints v1 rounded to 3.5 m/s and then re-uses that rounded value in every later hand calculation (Kv, mu, ...), whereas this implementation carries full double precision through the whole chain, per the standard's own recommendation in clause 4.1 ("enough significant digits (at least double precision) be used ... to obtain an accurate result") and Annex C's own Note 1 ("calculation results ... may not round off exactly to the equations as displayed"). All other deviations are below 0.4% and attributable to the same effect.

| Symbol | Expected (Annex C) | Calculated | Diff % | Result |
| --- | --- | --- | --- | --- |
| beta1 (deg) | 84.00000 | 84.00000 | 0.00 | PASS |
| d1 (mm) | 38.26700 | 38.26709 | 0.00 | PASS |
| d2 (mm) | 120.66090 | 120.66099 | 0.00 | PASS |
| px1 (mm) | 12.64000 | 12.63559 | -0.03 | PASS |
| a (mm) | 79.46000 | 79.46404 | 0.01 | PASS |
| u | 30.00000 | 30.00000 | 0.00 | PASS |
| v1 (m/s) | 3.50000 | 3.52572 | 0.73 | PASS |
| n2 (rpm) | 58.33000 | 58.33333 | 0.01 | PASS |
| be2 (mm) | 25.51000 | 25.51139 | 0.01 | PASS |
| lf2 (mm) | 26.88000 | 26.87926 | 0.00 | PASS |
| Ks | 1000.00000 | 1000.00000 | 0.00 | PASS |
| Km | 0.82362 | 0.82362 | 0.00 | PASS |
| Kv | 0.30713 | 0.30713 | 0.00 | PASS |
| Kn | 0.45298 | 0.45298 | 0.00 | PASS |
| mu | 0.02169 | 0.02160 | -0.41 | PASS |
| FTw2 (N) | 4324.30000 | 4324.25448 | 0.00 | PASS |
| FFw2 (N) | 100.30000 | 99.95010 | -0.35 | PASS |
| POw (kW) | 1.59400 | 1.59365 | -0.02 | PASS |
| PFw (kW) | 0.35200 | 0.35240 | 0.11 | PASS |
| PIw (kW) | 1.94600 | 1.94605 | 0.00 | PASS |
| Tw2 (Nm) | 260.90000 | 260.88442 | -0.01 | PASS |
| eta_z1-2 | 0.81800 | 0.81856 | 0.07 | PASS |
| eta_z2-1 | 0.77800 | 0.77941 | 0.18 | PASS |
| sigma_AB (N/mm2) | 69.00000 | 69.00000 | 0.00 | PASS |
| FTB2 (N) | 12097.80000 | 12097.79607 | 0.00 | PASS |
| TB2 (Nm) | 729.90000 | 729.86605 | 0.00 | PASS |
| PIB (kW) | 5.44700 | 5.44677 | 0.00 | PASS |

Result: 27 of 27 checks passed. No warnings were raised for the Annex C input set (it is within every clause 1.2 scope limit).

The self-test can be reproduced at any time from a checkout of the repository:

dotnet run --project AGMA6134C21Calc/AGMA6134C21Calc.csproj -- --selftest

# 8. Using the calculator

- Open AGMAStandards.sln and run AGMAStandardsHub, or run AGMA6134C21Calc directly; it opens with the Annex C example already loaded in the PropertyGrid.

- The left PropertyGrid panel groups inputs into six categories: mesh geometry & speed, materials, lubrication, overhung load (optional), service factor, and lubricant viscosity band.

- "Calculate" runs the full clause 4-12 chain and prints a labeled, equation-referenced report (mirroring the Annex C layout) in the right-hand panel.

- "Load Example" resets every input back to the Annex C worked example.

- "Save Report..." writes the report text to a .txt file.

- Warnings (scope violations, or an equivalent input power that exceeds the rated input power) are shown in the status bar in red; a calculation with no warnings shows a green confirmation.

# 9. File manifest

| File | Purpose |
| --- | --- |
| AGMA6134C21Calc/AGMA6134C21Calc.csproj | Project file (net8.0-windows, WinForms) |
| AGMA6134C21Calc/Program.cs | Entry point; "--selftest" launches SelfTest instead of the UI |
| AGMA6134C21Calc/MainForm.cs | UI logic: run/load-example/save-report |
| AGMA6134C21Calc/MainForm.Designer.cs | UI layout (PropertyGrid + report TextBox + buttons) |
| AGMA6134C21Calc/Models/Enums.cs | GearCastingMethod, WormHeatTreat, OilType, OverhungDriveType, ServiceDuration, ShockLoadClass, AmbientTemperatureBand |
| AGMA6134C21Calc/Models/AgmaInputs.cs | All calculator inputs, defaulted to the Annex C example |
| AGMA6134C21Calc/Calc/MathUtil.cs | Degree-based trig helpers; radian-argument arcsine for Eq 12 |
| AGMA6134C21Calc/Calc/CalcTrace.cs | Calculation audit trail / report builder |
| AGMA6134C21Calc/Calc/AgmaCalculator.cs | The rating engine: clauses 4, 5, 6, 7.1.4, 10.1, 11.1, 12 |
| AGMA6134C21Calc/Calc/SelfTest.cs | Annex C validation harness (27 checks, all passing) |
| AGMAStandardsHub/StandardCatalog.cs | Adds the "ANSI/AGMA 6134-C21" entry to the hub |
| AGMAStandardsHub/AGMAStandardsHub.csproj | Adds the AGMA6134C21Calc project reference |
| AGMAStandards.sln | Adds the AGMA6134C21Calc project and build configurations |